CONTENTS

TITLE

Product Overview

I.LCD screenParameters

II.MotherboardSpecifications

三, ProductFunction

四、Power Parameters

五、Environmental Factors

六、Random Accessories

七、Packaging and Weight

八、Dimensions

ProductOverview

This series of products belongs toWall-mounted network versionadvertising machine, the whole body is made of metal cold-rolled steel plate with spraying, aluminum alloy profiles for the edge strips, 4MM special tempered glass for the panel, high-definition effect, which can be widely used in building lobbies,Elevators,Hospitals, stations, hotels, subways, shopping malls, government buildings and other occasions are mainly used for multimedia information disclosure, picture advertising, text, audio and video media playback. It is a high-end digital signage advertising product.

1.LCD screenParameters

Effect picture

| Product model | PH100D | PH133D | PH156D |
| --- | --- | --- | --- |
| Display size | 10.1 inch | 13.3 inch | 15.6 inch |
| Display ratio | （16：9） |  |  |
| Appearance color | Silver frame black body |  |  |
| Display type | LCD LCD screen |  |  |
| Picture display size(H×V) mm | 216.58*135.36 | 293.76 *165.24 | 342.16 * 191.59 |
| Physical resolution | 1280*800 | 1920*1080 |  |
| Display color | 16.7M（8bit） |  |  |
| Refresh rate | 60Hz |  |  |
| Brightness | 250cd/m2 |  |  |
| Contrast ratio | 1300:1 |  |  |
| Viewing angle (degrees) | 85/85/85/85 (Typ.)(CR≥10) |  |  |
| Display protection | 4mm fully tempered high explosion-proof glass |  |  |
| Backlight life | ＞50000 hours |  |  |

2.MotherboardSpecifications

(Adapt the motherboard according to the parameters of the screen）

| CPU | Quad-core ARM Cortex-A7, main frequency up to 1.2GHz |
| --- | --- |
| Memory | 1G DDR3 |
| Storage | 8GB |
| Decoding resolution | Maximum support1080P@60fps |
| Audio and video output | Left and right channel output (2*10W speakers) |
| System upgrade | SupportUSB upgrade |
| USB interface | 2 |
| RJ45 interface | 1 |

- 3.Product functions

| Local operation functions |  |
| --- | --- |
| Network settings | Support wired and wireless network connections |
| Time-sharing settings | Support time-sharing setting of advertising machine power on and off time |
| Language switching | Support Simplified Chinese and English switching |
| Screen conversion | Support 0°, 90°, 180°, 270° screen switching |
| System upgrade | Support system background upgrade and local upgrade |
| USB | Support USB insertion to update programs |
| Background publishing system functions |  |
| Program management | Support setting the program list, you can delete, preview, publish and edit the program, and retransmit with one click |
|  | Support setting the program's playback schedule |
|  | Support setting the message list, you can delete, preview, and edit the message |
| Terminal management | Support terminal monitoring and terminal settings for selected terminals. Monitor the screen, view historical screenshots, view terminal storage programs, perform formatting operations on the terminal, and remote operation of the terminal |
| Message production | Support synchronously setting message content, message playback duration, and message playback time |
| Program production | Support synchronously setting program name, program content, program duration, playback type, and playback time |

- 4.Power Parameters

| Input power | 100-240V~50/60Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Overall power consumption | ≤25W | ≤30W | ≤35W | ≤45W | ≤55W |
| Standby power consumption | ≤0.5W |  |  |  |  |

- 5.Environmental Factors

| Operating temperature | 0℃~60℃ |
| --- | --- |
| Operating humidity | 10%~75%RH |
| Storage temperature | 15℃~45℃ |
| Storage humidity | 10%~90%RH |
| Altitude | Below 5000 meters |

- 6.Random Accessories

| Remote control | 1 |
| --- | --- |
| Key | 1 |
| Warranty card | 1 |
| Certificate of conformity | 1 |

- 7.Packaging and Weight

| Package size(cm) | 300*240*100 | 385*280*110mm | 450*310*110mm |  |  |
| --- | --- | --- | --- | --- | --- |
| 净重（kg） |  |  |  |  |  |
| Gross weight（kg） |  |  |  |  |  |

- 8.Dimensions

| 10.1 inch size chart |  |
| --- | --- |
| 13.3 inch size chart |  |
| 15.6 inch size chart |  |