Contents (CONTENTS)

Title (TITLE)

Product Overview

1. LCD Screen Parameters

2. Mainboard Specifications

3. Product Functions

4. Power Supply Specifications

5. Environmental Factors

6. Included Accessories

7. Packaging and Weight

8. Dimensional Specifications

Product Overview

This series of products consists of wall-mounted network advertising displays, constructed with full-body cold-rolled steel plates coated by spraying, aluminum alloy profile edging, and a 4mm specialized tempered glass panel, offering high-definition display quality. They are widely applicable in building lobbies, elevators, hospitals, stations, hotels, subways, shopping malls, government buildings, and similar environments. Primarily designed for multimedia information display, image advertising, text, and audio-video playback, this is a high-end digital signage advertising product.

1. LCD Screen Parameters

Rendering

| Product Model | PH185D | PH195D | PH215D | PH240D | PH270D |
| --- | --- | --- | --- | --- | --- |
| Display Size | 18.5 inches | 19.5 inches | 21.5 inches | 24 inches | 27 inches |
| Aspect Ratio | （16：9） |  |  |  |  |
| Appearance Color | Silver Frame with Black Housing |  |  |  |  |
| Display Type | LCD Liquid Crystal Display |  |  |  |  |
| Active Display Area (H×V) mm | 409*230 | 432.5*235.5 | 476*268 | 522*294 | 622.5*343 |
| Physical Resolution | 1366*768 | 1920*1080 |  |  |  |
| Display Colors | 16.7M（8bit） |  |  |  |  |
| Refresh Rate | 60Hz |  |  |  |  |
| Brightness | 250cd/m2 |  |  |  |  |
| Contrast | 1000:1 | 1000:1 | 1000:1 | 1000:1 | 1000:1 |
| Viewing Angle (°) | 89/89/89/89 (Typ.)(CR≥10) |  |  |  |  |
| Display Protection | 4mm Fully Tempered, High Explosion-Resistance Glass |  |  |  |  |
| Backlight Life | >50,000 hours |  |  |  |  |

2. Mainboard Specifications

| CPU | Quad-core ARM Cortex-A7, operating frequency up to 1.2 GHz |
| --- | --- |
| Memory | 1G DDR3 |
| Storage | 8GB |
| Decoding Resolution | Maximum support: 1080P@60fps |
| Audio/Video Output | Stereo output (2 × 10W speakers) |
| System Upgrade | Supports USB-based upgrades |
| USB Ports | 2 ports |
| RJ45 Interface | 1 unit |

- 3. Product Functions

| Local Operation Functions |  |
| --- | --- |
| Network Settings | Supports both wired and wireless network connections |
| Time-sharing Settings | Supports time-sharing configuration for advertising device power on/off intervals |
| Language Switching | Supports switching between Simplified Chinese and English |
| Screen Rotation | Supports screen rotation at 0°, 90°, 180°, and 270° |
| USB | Supports USB insertion for program updates |
| Backend Publishing System Functions |  |
| Program Management | Supports configuration of program lists with options to delete, preview, publish, edit, and one-click retransmission |
|  | Supports scheduling of program playback |
|  | Supports configuration of message lists with options to delete, preview, and edit messages |
| Terminal Management | Supports terminal monitoring and configuration for selected terminals. Monitoring screen, viewing historical screenshots, accessing terminal-stored programs, performing terminal formatting operations, and remote terminal control. |
| Message Creation | Supports synchronized configuration of message content, playback duration, and playback schedule. |
| Program Creation | Supports synchronized configuration of program name, content, duration, playback type, and playback schedule. |

- 4. Power Supply Specifications

| Input Power | 100-240V~50/60Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Total Power Consumption | ≤25W | ≤30W | ≤35W | ≤45W | ≤55W |
| Standby Power Consumption | ≤0.5W |  |  |  |  |

- 5. Environmental Factors

| Operating Temperature | 0℃~60℃ |
| --- | --- |
| Operating Humidity | 10%~75%RH |
| Storage Temperature | 15℃~45℃ |
| Storage Humidity | 10%~90%RH |
| Altitude | Below 5000 meters |

- 6. Included Accessories

| Remote Control | 1 unit |
| --- | --- |
| Key | 1 piece |
| Warranty Card | 1 sheet |
| Certificate of Conformity | 1 sheet |

- 7.Packaging and Weight

| Packaging Dimensions (cm) | 57*11.5*38 | 57*11.5*38 | 62*11*42 | 68*11*45 | 76*12.5*55 |
| --- | --- | --- | --- | --- | --- |
| Net Weight (kg) | 5.2 | 5.8 | 6.2 | 7.5 | 9.5 |
| Gross Weight (kg) | 6 | 6.5 | 8 | 8.6 | 11 |

- 8. Dimensional Specifications

| 18.5-inch Dimensional Drawing |  |
| --- | --- |
| 21.5-inch Dimensional Drawing |  |
| 24-inch Dimensional Drawing |  |
| 27-inch Dimensional Drawing |  |